Problem 1: Define geodesy as a science and describe concisely the three pillars of modern geodesy.

Geodesy is the science of measuring the size and shape of the earth including the gravitational field and the rotation of the earth.  Geodesy has “three pillars” which are used to represent the geometry, physics and rotation of the earth.  The three pillars of geodesy are the geometric shape of the earth, the gravitational field of the earth, and finally the rate and path of rotation of the earth.  5

Problem 2: What is a geodetic "reference system" and what a "reference frame"?

The geodetic reference system is defined as 'a complete reference system for positioning a point on the earth, including datum, co-ordinate description, co-ordinate system and possibly a projection' (CEN 1998). In geodesy there are two different reference systems, the terrestrial reference system, and the celestial reference system.  Both the terrestrial and the celestial reference systems are used in geodesy to create the reference frame which is the realization of the reference system. 4 (The main function of the reference system is the definition of the orientation, origin, and scale of the coordinate system. The definition given above is not consistent with the definition given by IERS. The systems are not 'used' to create the reference frames. The reference frames are realizations of the systems, which are determined  based on some kind of measurements.)

Problem 3: What are the two fundamental global geodetic reference systems and what physical quantity links these two systems together? What are the two conventional systems accepted by IAG and IAU, and what are the corresponding most accurate reference frames? What organization maintains the systems and frames, and where are the conventions published?

Two fundamental global geodetic reference systems are the celestial and the terrestrial reference systems, and both are linked together physically by the rotation of the earth.  The current reference system for the celestial body is the Conventional Celestial Reference System (CCRS) and it is maintained by the International Earth Rotation and Reference Systems Service (IERS.)  The terrestrial reference system is the Conventional Terrestrial Reference System (CTRS) and is also maintained by the IERS.  The data for the reference frames was then published in 1997 by the IERS in a work titled the International Celestial Reference Frame (ICRF.) 3 (The main aspect of the two fundamental systems is that one is an inertial system fixed to the inert space, and the other a rotating system fixed to the solid Earth. The two conventional systems accepted by IAG and IAU are the ICRS and the ITRS. The most accurate frames are the ICRF and ITRF). The conventions are published by IERS as the IERS Conventions.

Problem 4: For the International Terrestrial Reference Frame (ITRF):

- How is the ITRF given and published?

“The ITRF is realized by estimates of the coordinates and velocities of a set of stations observed by   VLBI, SLR, GPS and DORIS.” (The 3-D Global Model, p188) The data was published under the title ITRF and since the first publication in 1988 more than 10 versions have been published with updated information.  The current datum is the IRTF2005. (The cite text is not very accurate. The ITRF is a realization of the ITRS, and it is published as a list of coordinates at the reference epoch and constant velocities of a set of reference points.)

- How do reference coordinates for ITRF reference points change over time?

“The ITRF uses the same a rectangular coordinate system as other 3-D datum’s…)( The 3-D Global Model, p188)  The datums used are the NAD83 and the ITRF00; the reference points can change with the change of the center of mass and the orientation of the coordinate center; however, the coordinate systems changes very little over time (with the exception o f earthquake zones.) (In the reference frame, the coordinates of the reference points change linearly with time - hence the constant velocities associated with each reference point.)

- What is the general mathematical relationship between two versions of the ITRF?

X(t)NAD83=Tx(t)+[1+st]∙x(t)ITRF +Ɛz(t)∙y(t)ITRF-Ɛy(t)∙z(t)ITRF

X(t)NAD83=Ty(t)-Ɛz(t)∙x(t)ITRF +[1+s(t)]∙y(t)ITRF+Ɛx(t)∙z(t)ITRF

z(t)NAD83=Tz(t)+Ɛy(t)∙x(t)ITRF-Ɛx(t) ∙y(t)ITRF+[1+s(t)] ∙z(t)ITRF

(The general relationship is a Helmert Transformation, NAD83 is not an ITRF.)

- How can the coordinates of a point in the two systems be compared?

Convert the lat/long of the first coordinate system to Earth Centered Earth Fixed (ECEF), apply the dX,Y,Z scale factor.  Then using ECEF apply the new values to compute lat/long in the new system. (The  coordinates of a point given in two reference frames can be compared using the Helmert transformation between the two frames and taking into account the point velocities in the two different systems to compensate for different reference epochs.)

7

Problem 5: What are the mathematical shapes of the solid Earth that are related to the main phases in the development of geodesy since its beginning and what are the main parameters of these shapes that were the focus of research/observations in these phases?

The mathematical shape of the earth can be calculated by the equation for the geoid, as seen below;

The four phases in the development of the earth correspond to the completed equation above by first understanding the radius of a spherical Earth, followed by the rotational ellipsoid of the earth, followed by the gravitational field of a geoid, and lastly the dynamics of the Earth’s surface and relativistic models of the Earth system.  4.5 (The key parameters were (1) the radius of the sphere, (2) the oblateness of the ellipsoid, (3) the geoid, (4) the surface kinematics and the system dynamics.)

Problem 6: How is the geodetic reference ellipsoid for the Earth defined and what are the key parameters?

The geodetic reference ellipsoid for the Earth is defined by a set of X,Y,Z Cartesian coordinates.  The Z-axis coincides with the minor axis of the reference ellipsoid. The X-axis runs from the origin through a point on the equatorial plane at the zero meridian. The Y-axis is perpendicular to the X-axis on the equatorial plane.  The X,Y,Z coordinates are the key parameters for the reference for the definition of the reference ellipsoid. 3 (The key parameters are the two major half axis and the flattening.)

Problem 7: What are the main endogenic and exogenic processes impacting the shape of the Earth from periods of sub-seconds to millions of years?

“Endogenic processes are largely driven by internal convection and adjustments caused by the redistribution of heat and mass while exogenic process operate on the earth’s surface and are driven by solar energy with the aid of water.” (Bloom, Arthur, 2004)  Examples of endogenic processes are tectonics, volcanism, and isostacy.  Examples of exogenic processes are weathering and erosion, glaciations, and the oscillation of the waves. 4 (Important exogenic processes for geodesy include tides, and surface loading due to atmosphere, ocean, hydrology, and cryosphere).

30.5 out of 40: B (3.05)